HELP FILE “STAB”

- Double click on “STAB.exe” to start the program.

- Click on “Proceed”

- Choose gradation (For Highway Construction in India, few common gradations as per MoRTH has been provided. For any other custom gradation choose “Other”)

- Enter the number of stocks (any value between 2-5)

- Enter the “Number of Sieves” (It can be any number depending on the specific gradation)

- Enter the “lower bounds” and “upper bounds” values as per the outlined specification.

- For the chosen number of stockpiles enter the weight retained based on sieve size distribution in laboratory

- Also include the weight retained on “Pan”

- The image below shows an example of dense bituminous concrete (commonly used in India for surface courses)

- Hit “Calculate” and “Wait” [The black console window has been provided for “Windows version” indicating the progress of the program].

- Please note that the waiting time will vary depending on the working system and number of stockpiles. Please be patient and let the program complete.

- After the completion of data processing, “Number of Solutions” and “Best Solution” will be displayed. The “Best Solution” is obtained by minimizing the least square error corresponding to the mid-point gradation.

- A report file is also generated with “all the possible solutions”. The file can be found in the “reports” folder as Report.txt

- Please note that the possible solutions generated are sorted corresponding to the “errors” in ascending order.

It is recommended that the program be run on a system with atleast 4 GB RAM (for better speed).

For any further queries kindly contact:

Nikhil Saboo

Assistant Professor

Department of Civil Engineering

Indian Institute of Technology (BHU), Varanasi

India-221005

Email: nikhilsaboo.civ@iitbhu.ac.in